PPD Setup Plate

Congratulations on the purchase of your PPD setup, tweak, droop and multi position setup plate!

Your precision machined plate is made of high quality certified 6061 T6 aluminum, and is built to last a lifetime of trouble free use with basic care. Please read care section for tips on cleaning and keeping its bright finish.

Required tools

- 3mm hex driver. (recommend ball for faster loading)

- Thin droop gage such as an Integy.

- (Optional) Feeler gage set or .005 and .010 in. feeler gages.

Tips on using

- Before using, we recommend to take one of the 4mm cap screws, and a washer, stand on head end with washer on, and ca cement or shoe-goo the washer in place. This screw should be used on battery slot hold down with flat clamp and will keep the washer from dropping. All other clamps may remain on, and only require ½ turn to remove.

- Check bottom of chassis for high spots! Any high spot will distort the chassis once clamped, and will give undesirable results.

- Set the chassis onto plate with back toward lettering, check the chassis before clamping by pressing down with 1 finger in different spots. This will tell you how far it is out. We recommend a feeler gage set for this, to get real numbers.

- When clamping, start with center clamp in battery slots. There is no need to over tighten! Just a 3 finger snug will flatten the chassis, the pins keep it from moving even while bench testing.

- Please use the supplied washers. They will keep clamps from getting grooved.

- The tweak plate will remove as much twist from your chassis as possible…however, it does not fix bent bulkheads and badly warped chassis, you now have the tool to check that and quickly locate the problem. With the bulkhead and top deck screws 1 turn loose if chassis sits badly warped it likely wont come all the way back without replacement.

Care and cleaning

- Clean and remove light scratches with good quality liquid auto polish or metal polish such as Mothers billet polish. Use a fine scotch bright pad to remove light scratches. Larger scratches can be carefully flat filed off, just remember…NO HIGH SPOTS.

- Avoid harsh cleaners such as Simple Green, they can stain raw aluminum. For quick clean use spray auto detailer or soapy water, dry immediately.

- Do not expose to extreme heat or cold temperatures, and avoid sudden changes in temperature such as heat gun to a cold plate. Plate flatness is within .003 in and extreme temperatures can contribute to warp. Your plate is a precision tool and needs to be treated as such.

Check back for accessories and more options at www.ppdbillet.com ®